Worksheet 1.1

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: 20BCS-808/B

Semester: 4th Sem						Date of Performance: 20/02/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

# Addition of two 8bit numbers, sum 8 bits.

# 2. Task to be done:

# Write the 8085 Micro Processor program to calculate the addition of two 8bit numbers.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

- Load the first number from memory location 1050 to Memory.

- Move the content of memory to accumulator,

- Move the content of accumulator to register H.

- Load the second number from memory location 1051 to Memory.

- Then add the content of register H and accumulator using “ADD” instruction and storing result at accumulator.

- Store the addition of both number in the register location at 1052.

5. Description/ Code:

According to the Given Instruction In Manual

# ORG 1000H

LXI H,1000

MOV A,M

INX H

MOV B,M

MVI C,00

ADD B

JNC 000D

INR C

000D:	   INX H

MOV M,C

HLT

# ORG 1000H

# DB 11H, 20H

By Doing Experiment:

# ORG 1000H

LXI H,1050

MOV A,M

INX H

ADD M

INX H

MOV M,A

HLT

# ORG 1050H

# DB 11H, 29H

# 6. Result/Output/Writing Summary:

According to the Given Instruction In Manual:

By Doing Experiment Step By Step:

Learning outcomes (What I have learnt):

1. Learnt how to do the 8085-microprocessor programming.

2. Learnt how to add the two 8bit numbers with the carry.

3. Learnt how to add the two 8bits numbers without the carry.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |