Worksheet 1.2

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: 20BCS-809/A

Semester: 4th Sem						Date of Performance: 20/02/2022

Subject Name: MPI Lab	                               	Subject Code: 20CSP-253

# 1. Aim/Overview of the practical:

# Addition of two 16bit numbers, sum 8 bits.

# 2. Task to be done:

# Write the 8085 Micro Processor program to calculate the addition of two 16bit numbers.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

# Software Requirements:

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

# Hardware Requirements:

- Processor –Any suitable Processor

- Main Memory - 128 MB RAM

- Hard Disk –minimum 20 GB IDE Hard Disk

- Removable Drives–1.44 MB Floppy Disk Drive –52X IDE CD-ROM Drive

- PS/2HCL Keyboard and Mouse

4. Algorithm (For programming-based labs):

Algorithm:

- Load the first number pair from memory location 3000 and 3001 to HL Pair.

- Exchange The HL content to DE Pair.

- Load the first number pair from memory location 3002 and 3003 to HL Pair.

- Move the content of E register to accumulator.

- Add L register and store in Accumulator.

- Move the content of accumulator to L register.

- Move the content of D register to accumulator.

- Add H register and store in Accumulator.

- Move the content of accumulator to H register.

- Store the HL pair data in Memory address 3004 and 3005.

5. Description/ Code:

Example: 1

# ORG 2000H

LHLD 3000

XCHG

LHLD 3002

MOV A,E

ADD L

MOV L,A

MOV A,D

ADD H

MOV H,A

SHLD 1004

HLT

# ORG 3000H

# DB 12, 11, 11, 12

Example: 2

# ORG 2000H

LHLD 3000

XCHG

LHLD 3002

DAD D

SHLD 3004

HLT

# ORG 3000H

# DB 12,42,99,12

# 6. Result/Output/Writing Summary:

# Example: 1

Example: 2

Learning outcomes (What I have learnt):

1. Learnt how to do the 8085-microprocessor programming.

2. Learnt how to add the two 16bit numbers.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |