Worksheet 1.3

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: ON20BCS-809/A

Semester: 4th Sem						Date of Performance: 20/02/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

- Subtraction of two 8bit numbers along with considering borrow.

- Subtraction of two 16bit numbers along with considering borrow.

# 2. Task to be done:

# Write the 8085 Micro Processor program to calculate the subtraction of two 8bit as well as 16bit numbers.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

a) Subtraction of two 8bit number:

- Load the first number from memory location 3000 to Memory.

- Move the content of memory to accumulator,

- Increase the memory location of HL pair.

- Load the second number from memory location 3001 to Memory.

- Move the content of memory to B,

- Then subtract the B from the Accumulator and store in Accumulator.

- Move the content of Accumulator to memory M,

- Exit the program.

b) Subtraction of two 16bit number

- Load the 1st pair number from memory location 3000,3001 to HL pair.

- Exchange it with the DE pairs.

- Load the 2nd pair number from memory location 3002,3003 to HL pair.

- Move the content from E to accumulator.

- Subtract the L from accumulator.

- Move the content from accumulator to L register.

- Move the content from D to accumulator.

- Subtract the H from accumulator with the borrow.

- Move Accumulator to H register.

- Move the content of HL pair Register to memory location 1004,1005,

- Exit the program.

5. Description/ Code:

a) Subtraction of two 8bit number:

# ORG 2000H

LXI H,3000

MOV A,M

INX H

MOV B,M

SBB B

INX H

MOV M,A

HLT

# ORG 3000H

# DB 42, 34

- Subtraction of two 16bit number

# ORG 2000H

LHLD 3000

XCHG

LHLD 3002

MOV A,E

SUB L

MOV L,A

MOV A,D

SBB H

MOV H,A

SHLD 1004

HLT

# ORG 3000H

# DB 42, 35,34,21

# 6. Result/Output/Writing Summary:

# a) Subtraction of two 8bit number:

- Subtraction of two 16bit number:

Learning outcomes (What I have learnt):

1. Learnt how to do the 8085-microprocessor programming.

2. Learnt how to Subtract the two 8bit numbers with the carry.

3. Learnt how to Subtract the two 16bits numbers with the carry.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |