Worksheet 2.1

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: ON20BCS-809/A

Semester: 4th Sem						Date of Performance: 22/03/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

- 1's complement of 8-bit number.

- 2's complement of 8-bit number.

# 2. Task to be done:

# Write a 8085 Microprocessor program to find the 1’s and 2’s Complement of 8-bit number.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

Algorithm to find the 1’s and 2’s complement of 8-bit number:

- Load the number from memory location 2000 to accumulator.

- Calculate the 1’s complement using CMA. This will store the calculated 1’s complement into the accumulator.

- Store the 1’s complement value in to the 2001 memory location.

- To find the 2’s complement adds immediate 01 via ADI 01 into the 1’s complemented value which is stored in accumulator.

- Store the 2’s complement value at the 2002 memory location.

- End the execution using HLT

5. Description/ Code:

Program to find 1’s and 2’s complement of 8-bit number:

# ORG 1000H

LDA 2000

CMA

STA 2001

ADI 01

STA 2002

HLT

# ORG 2000H

# DB 55

# 6. Result/Output/Writing Summary:

# 1’s Complement:

# 2’s Complement:

Learning outcomes (What I have learnt):

- Learnt to find the 1’s complement of 8-bit number.

- Learn to find the 2’s complement if the number.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |