Worksheet 2.3 or 6

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: ON20BCS-809/A

Semester: 4th Sem						Date of Performance: 01/04/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

- Shift Left 8-bit number by 1bity.

- Shift Left 8-bit number by 2bit.

# 2. Task to be done:

# Write an 8085 Microprocessor program to shift left of 8-bit number by 1bt and 2bit.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

Algorithm to find left shift of 8-bit number by 1bit without Carry:

- Load the data to the Accumulator from 2000 memory address.

- Do the left shift using RLC.

- Store the 1bit left shifted value from Accumulator to 2001 memory location.

- End the execution using HLT.

Algorithm to find left shift of 8-bit number by 1bit with Carry:

- Load the data to the Accumulator from 2000 memory address.

- Set the carry flag as 1 using STC.

- Do the left shift with carry using RAL.

- Store the 1bit left shifted value from Accumulator to 2001 memory location.

- End the execution using HLT.

Algorithm to find left shift of 8-bit number by 2bit without Carry:

- Load the data to the Accumulator from 2000 memory address.

- Do the left shift with carry using RLC 2 times.

- Store the 1bit left shifted value from Accumulator to 2001 memory location.

- End the execution using HLT.

5. Description/ Code:

Program to find the left shift of 8-bit number by 1bit without carry:

# ORG 1000H

LDA 2000

RLC

STA 2001

HLT

# ORG 2000H

# DB 55

# Program to find the left shift of 8-bit number by 1bit with carry:

# ORG 1000H

LDA 2000

STC

RAL

STA 2001

HLT

# ORG 2000H

#DB 55

# Program to find the left shift of 8-bit number by 2bit without carry:

# ORG 1000H

LDA 2000

RLC

RLC

STA 2001

HLT

# ORG 2000H

#DB 55

# 6. Result/Output/Writing Summary:

Program to find the left shift of 8-bit number by 1bit without carry:

# Program to find the left shift of 8-bit number by 1bit with carry:

# Program to find the left shift of 8-bit number by 2bit without carry:

Learning outcomes (What I have learnt):

- Learnt to find the 1’s complement of 16-bit number.

- Learn to find the 2’s complement of 16-bit number.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |