Worksheet 3.1 or 8

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: ON20BCS-809/A

Semester: 4th Sem						Date of Performance: 08/04/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

- Mask the lower nibble of an 8-bit number.

- Mask the higher nibble of an 8-bit number.

# 2. Task to be done:

# Write a 8085 Microprocessor program to perform the lower nibble & higher nibble of 8-bit.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

Algorithm to mask the lower nibble of an 8-bit number:

- Load the 1st data to the Accumulator ‘A’ from 1000 memory address.

- Move the data from Accumulator ‘A’ to register B.

- Load the 2nd data to Accumulator ‘A’ from 1001 memory address which is F0.

- Do ANDing operation between Accumulator ‘A’ and Register B by Using ANA B.

- Store the ANDed value from Accumulator ‘A’ to 1002 memory location.

- End the execution using HLT.

OR,

- Load the data to Memory from 1000 address using Immediate Instruction LXI H, 1000.

- Move The data from Memory to Accumulator ‘A’.

- Perform Immediate ANDing operation in Accumulator ‘A’ with ‘F0’ by Using ANI F0 instruction.

- Store the ANDed value from Accumulator ‘A’ to 1002 memory location.

- End the execution using HLT.

Algorithm to mask the higher nibble of an 8-bit number:

- Load the 1st data to the Accumulator ‘A’ from 1000 memory address.

- Move the data from Accumulator ‘A’ to register B.

- Load the 2nd data to Accumulator ‘A’ from 1001 memory address which is 0F.

- Do ANDing operation between Accumulator ‘A’ and Register B by Using ANA B.

- Store the ANDed value from Accumulator ‘A’ to 1002 memory location.

- End the execution using HLT.

OR,

- Load the data to Memory from 1000 address using Immediate Instruction LXI H, 1000.

- Move The data from Memory to Accumulator ‘A’.

- Perform Immediate ANDing operation in Accumulator ‘A’ with ‘0F’ by Using ANI 0F instruction.

- Store the ANDed value from Accumulator ‘A’ to 1002 memory location.

- End the execution using HLT.

5. Description/ Code:

Program to mask the lower nibble of an 8-bit number:

# ORG 0900H

LDA 1000

MOV B, A

LDA 1001

ANA B

STA 1002

HLT

# ORG 1000

# DB D7H, F0H

OR

# ORG 0900H

LXI H, 1000

MOV A, M

ANI F0H

STA 1002

HLT

# ORG 1000

# DB D7H

# Program to mask the higher nibble of an 8-bit number:

# ORG 0900H

LDA 1000

MOV B, A

LDA 1001

ANA B

STA 1002

HLT

# ORG 1000

# DB D7H, 0FH

OR

# ORG 0900H

LXI H, 1000

MOV A, M

ANI 0FH

STA 1002

HLT

# ORG 1000

# DB D7H

# 6. Result/Output/Writing Summary:

Output to mask the lower nibble of an 8-bit number:

OR

# Program to mask the higher nibble of an 8-bit number:

# OR

Learning outcomes (What I have learnt):

- Learnt to mask the lower nibble of an 8-bit number.

- Learnt to mask the higher nibble of an 8-bit number.

- Learnt to perform the ANDing operation between two 8-bit number.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |