Worksheet 3.2 or 9

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: ON20BCS-809/A

Semester: 4th Sem						Date of Performance: 08/04/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

- Find the smaller out of two numbers.

- Find the larger out of two numbers.

# 2. Task to be done:

# Write an 8085 Microprocessor program to find the smaller & higher number out of two number.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

Algorithm to Find the smaller out of two numbers:

- Load the data to Memory from 1000 address using Immediate Instruction LXI H, 1000.

- Move The data from Memory 1000 to Accumulator ‘A’.

- Increment the HL pair using INX H.

- Move The data from Memory 1001 to Register ‘B’.

- Compare Register B with Accumulator using CMP B.

- Check if Carry flag generated using JC Instruction.

- If carry flag generated and set to 1 jump to the label and store the value of Accumulator ‘A’ to 1002 using STA 1002.

- If carry flag not generated then move the Register ‘B’ to Accumulator ‘A’ then store the value of Accumulator ‘A’ to 1002 using STA 1002.

- End the execution using HLT.

Algorithm to Find the larger out of two numbers:

- Load the data to Memory from 1000 address using Immediate Instruction LXI H, 1000.

- Move The data from Memory 1000 to Accumulator ‘A’.

- Increment the HL pair using INX H.

- Move The data from Memory 1001 to Register ‘B’.

- Compare Register B with Accumulator using CMP B.

- Check if Carry flag generated using JNC Instruction.

- If carry flag not generated jump to the label and store the value of Accumulator ‘A’ to 1002 using STA 1002.

- If carry flag generated and set to 1 then move the Register ‘B’ to Accumulator ‘A’ then store the value of Accumulator ‘A’ to 1002 using STA 1002.

- End the execution using HLT.

5. Description/ Code:

Program to Find the smaller out of two numbers:

# # ORG 0900H

# LXI H,1000

# MOV A, M

# INX H

# MOV B, M

# CMP B

# JC LABEL

# MOV A, B

# LABEL:	STA 1002

# HLT

# # ORG 1000

# # DB D7H, F6H

# Program to Find the larger out of two numbers:

# # ORG 0900H

# LXI H,1000

# MOV A, M

# INX H

# MOV B, M

# CMP B

# JNC LABEL

# MOV A, B

# LABEL:	STA 1002

# HLT

# # ORG 1000

# # DB D7H, F6H

# 6. Result/Output/Writing Summary:

Output to Find the smaller out of two numbers:

When 1st number is smaller number

When 2nd number is smaller number

# Output to Find the larger out of two numbers:

# When 2nd number is larger number

# When 1st number is larger number

# Learning outcomes (What I have learnt):

- Learnt to find larger number in two number.

- Learnt to find smaller number in two number.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |