Worksheet 3.3 or 10

Student Name: Vivek Kumar				UID: 21BCS8129

Branch: BE-CSE (LEET) 	                               Section/Group: ON20BCS-809/A

Semester: 4th Sem						Date of Performance: 22/04/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

- Find the smaller number in an array.

- Find the larger number in an array.

# 2. Task to be done:

# Write an 8085 Microprocessor program to find the smaller & larger number in an array.

# 3. Apparatus/Simulator used (For applied/experimental sciences/materials-based labs):

- 8085 Jubin simulator version 2 (Microprocessor Simulator)

- Java (jdk/ jre1.8.0_321)

4. Algorithm/Flowchart (For programming-based labs):

Algorithm to Find the smaller out of two numbers:

- Load the Counter data to Memory from 1000 address using Immediate Instruction LXI H, 1000.

- Move The data from Memory 1000 to Register ‘C’.

- Increment the HL pair using INX H which will access the data from next address.

- Move The data from Memory ‘M’ to Accumulator ‘A’.

- Decrease the counter value by 1 which is stored in Register ‘C’ using DCR C.

- Increment the HL pair using INX H which will access the data from next address.

- Compare Data stored in Memory ‘M’ with Accumulator using CMP M.

- Check if Carry flag generated using JC Instruction.

- If carry flag generated and set to 1 jump to the label and decrease the counter value by 1 which is stored in Register ‘C’ using DCR C.

- If carry flag not generated and set to 0. Move The data from Memory ‘M’ to Accumulator ‘A’ and then decrease the counter value by 1 which is stored in Register ‘C’ using DCR C.

- Now check with the Zero flag generated or not using JNZ instruction.

- If zero flag not generated and set to 0 jump to the loop and increment HL pair using INX H which will access the data from next address.

- Store the Accumulator data to the To the Memory address 2000 using STA 2000.

- End the execution using HLT.

Algorithm to Find the larger out of two numbers:

- Load the Counter data to Memory from 1000 address using Immediate Instruction LXI H, 1000.

- Move The data from Memory 1000 to Register ‘C’.

- Increment the HL pair using INX H which will access the data from next address.

- Move The data from Memory ‘M’ to Accumulator ‘A’.

- Decrease the counter value by 1 which is stored in Register ‘C’ using DCR C.

- Increment the HL pair using INX H which will access the data from next address.

- Compare Data stored in Memory ‘M’ with Accumulator using CMP M.

- Check if Carry flag not generated using JNC Instruction.

- If carry flag no generated and set to 0 jump to the label and decrease the counter value by 1 which is stored in Register ‘C’ using DCR C.

- If carry flag generated and set to 1. Move The data from Memory ‘M’ to Accumulator ‘A’ and then decrease the counter value by 1 which is stored in Register ‘C’ using DCR C.

- Now check with the Zero flag generated or not using JNZ instruction.

- If zero flag not generated and set to 0 jump to the loop and increment HL pair using INX H which will access the data from next address.

- Store the Accumulator data to the To the Memory address 2000 using STA 2000.

- End the execution using HLT.

5. Description/ Code:

Program to Find the smaller number in an array:

# # ORG 0900H

# LXI H,1000

# MOV C,M

# INX H

# MOV A,M

# DCR C

# LOOP:      INX H

# CMP M

# JC LABEL

# MOV A,M

# LABEL:    DCR C

# JNZ LOOP

# STA 2000

# HLT

# # ORG 1000

# # DB 06H,09H,F6H,09H,D5H,02H,10H

Program to Find the larger number in an array:

# # ORG 0900H

# LXI H,1000

# MOV C,M

# INX H

# MOV A,M

# DCR C

# LOOP:      INX H

# CMP M

# JNC LABEL

# MOV A,M

# LABEL:    DCR C

# JNZ LOOP

# STA 2000

# HLT

# # ORG 1000

# # DB 09H,01H,F6H,D0H,2DH,02H,F0H,FDH,A2H,F3H

# 6. Result/Output/Writing Summary:

Output to Find the smaller number in an array:

Now the loop will run Until our Zero flag set to 1. And then it will finally store the Accumulator value to 2000.

Output to Find the larger number in an array:

Now the loop will run Until our Zero flag set to 1. And then it will finally store the Accumulator value to 2000.

# Learning outcomes (What I have learnt):

- Learnt to find larger number in a given Array.

- Learnt to find smaller number in a given Array.

- Learnt to use the Nested Looping statement in the single program.

- Learnt to use the JC, JNC & JNZ instruction.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |
| 4. |  |  |  |